# PART 1 - GENERAL

  - SUMMARY

    - Division Includes:

      - Dry Erase Wallcovering.

      - Magnetic Receptive Dry Erase Wallcovering.

      - Projection Dry Erase Wallcovering.

      - Adhesive Backed Dry Erase Wallcovering.

      - Tray and Trim

      - Accessories.

    - Related Divisions:

      - Division 26 51 00Interior Lighting.

      - Division 09 20 00Plaster and Gypsum Board: Wall substrate.

      - Division 09 91 23Interior Painting: Priming for vinyl wall coverings.

      - Division 09 72 16.13 Flexible Vinyl Wall Coverings.

      - Division 10 11 00Visual Display Surfaces: Chalk and markerboards.

# REFERENCES

    - American Society for Testing and Materials (ASTM):

E84	Test Method for Surface Burning Characteristics of Building Materials.

    - Gypsum Association

GA-214-M-97	Recommended Levels of Gypsum Board Finish.

# SUBMITTALS

    - Manufacturer’s product data and installation instructions for each type of dry erase wallcovering, adhesive, and accessories required.

    - Manufacturer’s written product data indicating compliance with specified materials required.

    - Manufacturer’s written installation instructions.

    - Manufacturer’s written instructions for recommended maintenance of each type of dry erase wall covering required.

    - Samples:

      - 7 inch (177.8mm) x 9 inch (228.6mm) samples of each dry erase material required.

      - 6 inch (152.4mm) samples of trim, tray, and end caps required.

# QUALITY ASSURANCE

    - Manufacturer: Provide each type of dry erase wallcovering required produced by one manufacturer.

    - Installer: Installation by skilled commercial wallcovering contractor with no less than three years of documented experience installing dry erase wallcovering of the types and extent required.

    - Composition:

      - Nu-Vu-Rite™: Provide non-woven backing, pigmented vinyl capped with matte, projectable, dry erase film, embossed with a bi-directional lenticular pattern that meets California Indoor Air Quality Specification 01350.

      - Project-Rite™: Provide non-woven backing, pigmented vinyl capped with matte, projectable, dry erase and slight emboss that meets California Indoor Air Quality Specification 01350.

      - Matte-Rite™: Provide specific backing: non-woven or adhesive backing with white pigmented vinyl capped with matte, projectable, dry erase film that meets California Indoor Air Quality Specification 01350.

      - Projectable Mag-Rite™: Provide woven backing, ferrous sheet bonded with white pigmented vinyl capped with matte, projectable, dry erase film.

      - Mag-Rite®: Provide woven backing, ferrous sheet bonded with white pigmented vinyl and capped with semi-gloss, dry erase film that meets California Indoor Air Quality Specification 01350.

      - Mag-Rite® 59: Provide woven backed, ferrous sheet bonded with white pigmented vinyl and capped with semi-gloss, dry erase film that meets California Indoor Air Quality Specification 01350.

      - Just-Rite®: Provide woven backing, white pigmented vinyl capped with semi-gloss, dry erase film that meets California Indoor Air Quality Specification 01350. (Note: Just-Rite 60 meets California Indoor Air Quality Specification 01350; Just- Rite 48 has not been tested to that standard.)

      - EZ-Rite®: Provide specific backing: non-woven or adhesive backing, white pigmented vinyl capped with high-gloss, dry erase film that meets California Indoor Air Quality Specification 01350.

      - Koro-Rite®: Provide non-woven backing, pigmented vinyl capped with high-gloss, dry erase film that meets California Indoor Air Quality Specification 01350.

    - Surface Burning Characteristics Classification: Provide materials that meet Class I/A rating when tested in accordance with ASTM E84 for flame spread and smoke developed

    - Field Samples: Prepare field samples for architect’s review and establish requirements for seaming and finish trim.

      - Install sample panel of each type presentation wallcovering specified in area designated by architect.

      - Maintain corrected and approved samples to serve as a standard of performance for the project.

# PRODUCT DELIVERY, STORAGE, AND HANDLING

    - Deliver presentation wallcoverings to the project site in unbroken and undamaged original factory packaging and clearly labeled with the manufacturer’s identification label, quality or grade, and lot number.

    - Store materials in a clean, dry storage area with temperature maintained above 55°F (13°C) with normal humidity.

    - Store material within original packaging to prevent damage.

# PROJECT CONDITIONS

    - Do not apply presentation wallcoverings when surface and ambient temperatures are outside the temperature ranges required by the wallcovering manufacturer.

    - Provide continuous ventilation and heating facilities to maintain substrate surface and ambient temperatures above 55°F (13°C) unless required otherwise by manufacturer’s instructions.

    - Apply adhesive when substrate surface temperature and ambient temperature is above 55°F (13°C) and relative humidity is below forty percent.

    - Maintain constant recommended temperature and humidity for at least 72 hours prior to and throughout the installation period, and for 72 hours after wallcovering installation completion.

    - Provide not less than 80-foot-candles per square foot lighting level measured mid-height at substrate surfaces.

# WARRANTY

    - Submit manufacturer’s limited ten-year written warranty against manufacturing defects.

# MAINTENANCE

    - Maintenance instructions: Include precautions against cleaning materials and methods that may be detrimental to finishes and performance.

# PART 2 - PRODUCTS

  - MANUFACTURER

    - Wallcoverings: Walltalkers Wallcoverings manufactured by Koroseal Interior Products, LLC., Fairlawn, Ohio, and distributed by. Contact sales representative.

# MATERIALS

    - Walltalkers Nu-Vu-Rite: Non-woven backed, pigmented vinyl capped with matte, projectable, dry erase film, embossed with a bi-directional lenticular pattern. Patent # 5,361,164.

      - NV60: 59/60 inch (1.50/1.52m) width, non-woven backing.

    - Walltalkers Project-Rite: Non-woven backed, pigmented vinyl capped with matte, projectable, dry erase and slight emboss.

      - PE50: 49/50 inch (1.25/1.27m) width, non-woven backing, white only.

      - PE60: 59/60 inch (1.50/1.52m) width, non-woven backing, specify color.

    - Walltalkers Matte-Rite: Specify backing: non-woven or adhesive backing with white pigmented vinyl capped with matte, projectable, dry erase film.

      - MP50: 49/50 inch (1.25/1.27m) width, non-woven backing, white only.

      - MP60: 59/60 inch (1.50/1.52m) width, non-woven backing, white only

      - MA50: 49/50 inch (1.25/1.27m) width, self-adhesive backing, white only.

    - Walltalkers Projectable Mag-Rite: Woven backed, ferrous sheet bonded with white pigmented vinyl capped with matte, projectable, dry erase film.

      - M2PR: 47/48 inch (1.19/1.22m) width, scrim backing.

    - Walltalkers Mag-Rite: Woven backed, ferrous sheet bonded with white pigmented vinyl and capped with semi-gloss, dry erase film.

      - M248: 47/48 inch (1.19/1.22m) width, woven backing.

      - M259: 59/60 inch (1.50/1.52m) wide, woven backing.

    - Walltalkers Just-Rite: Woven backed, white pigmented vinyl capped with semi-gloss, dry erase film.

      - JR48: 47/48 inch width (1.19/1.22m), scrim backing.

      - JR60: 59/60 inch (1.50/1.52m) width, scrim backing.

    - Walltalkers EZ-Rite: Specify backing: non-woven or self-adhesive backing, white pigmented vinyl capped with high-gloss, dry erase film

      - ZA50: 49/50 inch (1.25/1.27m) width, self-adhesive backing.

      - ZP50: 49/50 inch (1.25/1.27m) width, non-woven backing.

    - Walltalkers Koro-Rite: Non-woven backed, pigmented vinyl capped with high-gloss, dry erase film.

      - KR60: 59/60-inch (1.50/1.52m), non-woven backing, white only

# TRIM & TRAY

    - Aluminum Tray: Clear satin, anodized aluminum, snap-on marker and eraser tray with clips. Specify length: 4’ length (TY04-00), 8’ length (TY08-00), 12’ length (TY12-00)

    - Aluminum Trim: Clear satin, anodized aluminum, snap-on trim with clips. Specify length: 4’ length (TY04-00), 8’ length (TY08-00), 12’ length (TY12-00)

    - End Caps:

      - ET02-00: 1/4 inch (6mm) box tray end cap set for marker and eraser tray.

      - ET03-00: 1/2 inch (13mm) anodized tray end cap set for marker and eraser tray.

    - J Cap Wallcovering Trim: JC12-00: Clear satin, anodized aluminum, low profile trim Also available in Black and White powder coat

    - Quantum Tray: Aluminum blade style marker and eraser tray with angled, smooth-finished ends and installation hardware kit. Specify finish and length:

      - QA18TY-00: Silver anodized aluminum, 18 inch (457 mm) length only.

      - QA36TY-00: Silver anodized aluminum, 36 inch (914 mm) length only.

      - QA48TY-00: Silver anodized aluminum, 48 inch (1.22 m) length only.

      - QP18TY-05: White powder coated aluminum, 18 inch (457 mm) length only.

      - QP36TY-05: White powder coated aluminum, 36 inch (914 mm) length only.

      - QP48TY-05: White powder coated aluminum, 48 inch (1.22 m) length only.

      - QP18TY-04: Black powder coated aluminum, 18 inch (457 mm) length only.

      - QP36TY-04: Black powder coated aluminum, 36 inch (914 mm) length only.

      - QP48TY-04: Black powder coated aluminum, 48 inch (1.22 m) length only.

    - Quantum Mag Tray: Aluminum blade style marker and eraser tray with magnetic backing. Specify finish:

      - QM18TY-00: Silver anodized aluminum, 18 inch (457 mm) length only.

      - QM36TY-00: Silver anodized aluminum, 36 inch (914 mm) length only.

      - QM48TY-00: Silver anodized aluminum, 48 inch (1.22 m) length only.

      - QM18TY-05: White powder coated aluminum, 18 inch (457 mm) length only.

      - QM36TY-05: White powder coated aluminum, 36 inch (914 mm) length only.

      - QM48TY-05: White powder coated aluminum, 48 inch (1.22 m) length only.

      - QM18TY-04: Black powder coated aluminum, 18 inch (457 mm) length only.

      - QM36TY-04: Black powder coated aluminum, 36 inch (914 mm) length only.

      - QM48TY-04: Black powder coated aluminum, 48 inch (1.22 m) length only.

# ACCESSORIES

    - Adhesives: Heavy-duty clear or clay based premixed vinyl adhesive.

    - Substrate Primer/Sealer: White pigmented acrylic base primer/sealer specifically formulated for use with vinyl wallcoverings.

    - Aluminum Marker Caddy: Provide 3 inch (76 mm) high x 7-1/4 inch (184 mm) wide aluminum caddy with four holes and a slot for storing markers and an eraser.

      - AMC1-00: Aluminum marker & eraser caddy, silver anodized aluminum.

      - AMC1-05: Aluminum marker & eraser caddy, white powder coated aluminum.

      - AMC1-04: Aluminum marker & eraser caddy, black powder coated aluminum.

      - AMCM-00: Aluminum marker & eraser caddy w/ magnetic backing, silver anodized aluminum.

# PART 3 - EXECUTION

  - EXAMINATION

    - Examine substrates and installation conditions to ensure surface conditions meet or exceed a Level 4 finish, per GA- 214-M-97: Recommended Levels of Gypsum Board Finish, and permanent lighting should be installed and operational.

    - Test substrate with suitable moisture meter and verify that moisture content does not exceed four percent.

    - Verify substrate surface is clean, dry, smooth, structurally sound, and free from surface defects and imperfections that would show through the finished surface.

    - Evaluate all painted surfaces for the possibility of pigment bleed-through.

    - Notify the contractor and architect in writing of any conditions detrimental to the proper and timely completion of the installation.

    - Beginning of installation means acceptance of surface conditions.

# INSTALLATION: WALLCOVERING BACKING

    - Acclimate wallcovering in the area of installation a minimum of 24 hours before installation.

    - Read and follow the manufacturer’s installation instruction sheet contained in each roll of the dry erase wallcovering.

    - Examine all materials for pattern, color, quantity and quality, as specified for the correct location prior to cutting.

    - Primer: Use a quality pigmented acrylic wallcovering primer.

    - Adhesive: Apply a uniform coat of heavy-duty pre-mixed clay-based or extra strength clear wallcovering adhesive.

    - Install each strip horizontally and in the same sequence as cut from the roll.

    - Install dry erase wallcovering sheets in exact order as they are cut from bolt. Reverse hang alternate strips (except lined products). Do not crease or bend the wallcovering when handling.

    - Install dry erase wallcovering horizontally using a level line.

    - Using a level or straight edge, double cut the seam with a seam-cutting tool (Ex: Double Seam-Cutter or Swedish Knife). Do not score drywall or plasterboard when cutting material.

    - When covering the entire wall, seam the material out of the main writing and viewing areas of the wall.

    - Apply wallcovering to the substrate using a wallcovering smoother, wrapped with a soft cloth, to remove air bubbles. Do not use sharp edged smoothing tools. Smooth material on the wall from the middle to the outside edge.

    - Remove excess adhesive immediately after the wallcovering is applied. Clean entire surface with a warm mild soap solution, and clean soft cloths. Rinse thoroughly with water and let dry before using. Change water often to maintain water clarity.

    - Stop installation of material that is questionable in appearance and notify the manufacturer’s representative for an inspection.

# INSTALLATION: SELF-ADHESIVE BACKING

    - Apply Walltalkers adhesive backed dry erase wallcovering only on surfaces impervious to moisture such as chalkboards, marker boards, glass, high-pressure laminates, or similar.

    - Acclimate wallcovering in the area of installation a minimum of twenty-four hours before installation.

    - Examine all materials for color, quantity, and quality as specified for the correct location prior to cutting.

    - Read and follow the instructions in the manufacturer’s installation sheet contained in each roll of the dry erase wallcovering.

    - Do not crease or bend the wallcovering when handling.

    - Mix dampening solution by using one half to one capful of mild detergent to 1 gallon (1.81kg) clean water. Damping solution is used in positioning the material and allows for the removal of air bubbles.

    - Use a pump spray bottle to apply the dampening solution to the the surface.

    - Slowly remove release liner and smooth wall covering to the hanging surface using a wallcovering smoother wrapped with a soft cloth from the middle to the outside edge to remove air bubbles.

    - Stop installation of material that is questionable in appearance and notify the manufacturer’s representative for an inspection.

# CLEAN-UP

    - Upon completion of installation, remove all exposed adhesive immediately using a soft cloth and a warm, mild soap solution and rinse thoroughly with water and dry with clean towel prior to using.

    - Upon completion of the work, remove surplus materials, rubbish, and debris resulting from the wallcovering installation. Leave areas in neat, clean, and orderly condition.

Matte-Rite™, EZ-Rite®, Just-Rite®, Koro-Rite®, Mag-Rite®, Tac-Wall®, Walltalkers®, Projectable Mag-Rite™, Project-Rite™, and Nu-Vu-Rite™ are registered trademarks and trademarks of Koroseal Interior Products, LLC.

PresentationMate™ is a trademark of Strategic Synergies, Inc.

UPDATED: OCTOBER 2022